Character Windows

TWIN Adaptations (CW 2.20.01)

Scott Randell

Microsoft Applications Division

7/31/26

.c.1. Introduction

This appendix describes the variations of CW that exist to emulate TWIN (CW's predecessor) functionality.  These are of interest to the Microsoft Language products (QuickBasic and QuickC).

These variations are made at compile time (in the CW version file).

.c.2. Cosmetic features

The following cosmetic features exist in TWIN compatible versions of CW:

* Buttons are 3 chararacters high and look different.  The default button is drawn with a double wide border.

* There is no line at the bottom of dialogs.

.c.3. Functional Changes

.c.:3.1 Accelerators

Accelerators are possible without a corresponding menu item.

Accelerators in menus and dialog can be non-unique.

.c.4. One High Listboxes

Single character high listboxes (3 high including the border) are allowed (they do not have a scroll bar).  Please note that showing the listbox selection is visually different than real TWIN.

.c.:4.1 Horizontal Listboxes

Listboxes scroll horizontally (instead of the standard CW vertical listbox).  The user interface is significantly different.  The API has 1 additional call SetTmcListWidth() -- see below.

.c.5. API changes

.c.:5.1 SetTmcListWidth

For horizontally scrollable listboxes this sets the number of columns for the listbox.  This should be called for listboxes of more than 1 column in response to the dlmInit message to the dialog proc.  Listboxes that are more than 1 column wide must therefore have a dialog proc.

Listboxes that are 1 column wide do not need to call this procedure.  The Listbox handler automatically determines the width of each column.

VOID SetTmcListWidth(tmc, citem)

TMC	tmc;

WORD	citem;

tmc is the item code of the listbox.  citem is the number of items wide the listbox should be (i.e. the number of columns of items that are visible at 1 time).

Table Of Contents

.Begin Table C.

1. Introduction	1

2. Cosmetic features	1

3. Functional Changes	1

3.1 Accelerators	1

4. One High Listboxes	1

4.1 Horizontal Listboxes	1

5. API changes	2

5.1 SetTmcListWidth	2

.End Table C.